MID TERM EXAMINATION, SPRING 2008 C.E- 308 CEP & TECHNICAL REPORT WRITING

6"' SEMESTER CIVIL ENGINEERING

Til

Time: - 2Hour

Marks:-80

Note: - Attempt all questions. All question carry equal remarks. Assume any suitable data if missing.

(2)

For the given plan (Fig. 1) estimate the quantities of

i.      Earthwork in excavation in foundation ii.       Cement concrete in foundation iii.       Brick work in foundation & plinth iv.      D.P.C

Prepare a detailed estimate of a R.C.C beam of a 14 11 clear span 12 in x 18 in in crass section from the given drawing (Fig 2). Steel in detail and R.C.C work shall be calculated separately

(31

A road is to be constructed at a uniformly upward gradient of_l in 200. The

>«       '.   1

natural surface levels are as follows.

| Chainage | 0 | 100 | 200 | 300 | 400 | 500 |
| --- | --- | --- | --- | --- | --- | --- |
| N'.S.L (ft) | 200.00 | 200.70 | 202.20 | 204.10 | 205.60 | 207.20 |

(4)

If the formation level at 0 chainage is 201.80 ft, estimate the volume of filling and cutting in cu ft for a formation width of 24 ft. The side slope in cutting are 1:1 & side slope in filling are 2:1 (a) Define estimate. Write the types of Approximate cost estimates for

building, (b)) Write note on the following

(i) Revised Estimate	(ii) Supplementary & Revised Estimate

(iii) Annual Repair or Maintenance Estimate

I —