NWFP University of Engineering & Technology Peshawar, Pakistan.

Mid [Term Examination, 2nd semester spring 2006

CE-104 Mechanics of Solids-1

Maximum Marks = 80

Note:	Time Allowed = 2hrs

1.   All Questions are compulsory and carrying equal Marks 2._  Text Book, Class Notes, and any Programmable calculator are not allowed.

Q)-2

a): Two blocks of wood, width w and thickness t, are glued together along the joint inclined at the angle 6 as shown in the figure. Using the free-body diagram concept, show that the shearing stress on the glued joint is

t = Psin28/2A,	Where "A" is the cross-sectional area.

b): An aluminum rod is rigidly attached between a steel rod and a bronze rod as shown in figure. Axial loads are applied at the positions indicated. Find the maximum value of P that will not exceed a stress in steel of HOMPa, in aluminum of 90MPa or in bronze of lOOMPa.

Q)-3

a): State Hooke's Law and Prove the relation as follow,

Where

P= Axial Load, L-length of member, A=x-sectional Area of member, E=modulus of Elasticity

b): The rigid bar ABC shown below is hinged at A and supported by a steel rod at 0. Determine the largest load P that can be applied at C if the vertical movement of end C must not exceed 0.10 in.

a): Prove that,

ot = aEAT, ot= Thermal stress a = coefficient of linear expansion E = Modulus of Elasticity AT = Change in Temperature

b): A bronze bar 3m long with a cross-sectional Area of 320 mm is placed between two r igid w alls as s hown i n t he figure. A t a t emperature o f - 20 ° C, t he gap i s 2.5mm. Find the temperature at which the compressive stress in the bar will be

35 MPa. Use a - 18*10A-6 m/m. °C and E = 80 GPa.

Q)-5"

a): A specimen of any given material is subjected to a uniform tri-axial stress. Determine the theoretical maximum value of poison's ratio.

b): The compound shaft shown below is attached to rigid supports. For the bronze segment AB, the diameter is 75 mm, T < 60 MPa, and G = 35 GPa. For the steel segment BC, the diameter is 50 mm, t < 80 MPa, and G = 83 GPa. Compute the maximum   torque T that can be applied.